Saveetha School of Engineering

Saveetha Institute of Medical and Technical Sciences

CSA12 - Computer Architecture

INDEX

| Exp.No. | TITLE | Page no. | Sign. |
| --- | --- | --- | --- |
| 1. | 8-BIT ADDITION | 5 |  |
| 2. | 8-BIT SUBTRACTION | 6 |  |
| 3. | 8-BIT MULTIPLICATION | 7 |  |
| 4. | 8-BIT DIVISION | 8 |  |
| 5. | 16 -BIT ADDITION | 9 |  |
| 6. | 16-BIT SUBTRACTION | 10 |  |
| 7. | 16-BIT MULTIPLICATION | 11 |  |
| 8. | 16-BIT DIVISION | 12 |  |
| 9. | FACTORIAL OF A GIVEN NUMBER | 13 |  |
| 10. | LARGEST NUMBER IN AN ARRAY | 14 |  |
| 11. | SMALLEST NUMBER IN AN ARRAY | 15 |  |
| 12. | ASCENDING ORDER | 16 |  |
| 13. | DESCENDING ORDER | 17 |  |
| 14. | ADDITION OF N NUMBERS | 18 |  |
| 15. | SWAPPING OF NUMBERS | 19 |  |
| 16. | SQUARE OF NUMBER | 20 |  |
| 17. | ONEs AND TWOs COMPLEMENT | 21 |  |
| 18. | ROTATE LEFT OPERATION | 22 |  |
| 19 | ROTATE RIGHT OPERATION | 23 |  |
| 20 | LOGICAL OPERATIONS | 24 |  |
| 21 | DECIMAL TO BINARY CONVERSION | 25 |  |
| 22 | HEXA DECIMAL TO DECIMAL CONVERSION | 26 |  |
| 23 | DECIMAL TO OCTAL CONVERSION | 27 |  |
| 24 | BINARY TO DECIMAL CONVERSION | 28 |  |
| 25 | TWO STAGE PIPELINE | 29 |  |
| 26 PERFORMANCE | CPU PERFORMANCE | 30 |  |
| 27 | TWO BIT HALF ADDER | 31 |  |
| 28 | TWO BIT HALF SUBTRACTOR | 32 |  |
| 29 | FULL ADDER | 33 |  |
| 30 | FULL SUBTRACTOR | 34 |  |

8-BIT ADDITION

EXP NO: 1

AIM:

To write an assembly language program to implement 8-bit addition using 8085 processor.

ALGORITHM:

- Start the program by loading the first data into the accumulator.

- Move the data to a register.

- Get the second data and load it into the accumulator.

- Add the two register contents.

- Check for carry.

- Store the value of sum and carry in the memory location.

- Halt.

PROGRAM:

LDA 8500

MOV B, A

LDA 8501

ADD B

STA 8502

RST 1

INPUT:

OUTPUT:

RESULT: Thus the program was executed successfully using 8085 processor simulator.

8-BIT SUBTRACTION

EXP NO: 2

AIM: To write an assembly language program to implement 8-bit subtraction using 8085 processor.

ALGORITHM:

- Start the program by loading the first data into the accumulator.

- Move the data to a register.

- Get the second data and load it into the accumulator.

- Subtract the two register contents.

- Check for borrow.

- Store the difference and borrow in the memory location.

- Halt.

PROGRAM:

LDA 8000

MOV B, A

LDA 8001

SUB B

STA 8002

RST 1

INPUT:

OUTPUT:

RESULT: Thus the program was executed successfully using 8085 processor simulator.

8-BIT MULTIPLICATION

EXP NO: 3

AIM:

To write an assembly language program to implement 8-bit multiplication using 8085 processor.

ALGORITHM:

- Start the program by loading a register pair with the address of memory location.

- Move the data to a register.

- Get the second data and load it into the accumulator.

- Add the two register contents.

- Increment the value of the carry.

- Check whether the repeated addition is over.

- Store the value of product and the carry in the memory location.

- Halt.

PROGRAM:

LDA 8500

MOV B, A

LDA 8501

MOV C, A

CPI 00

JZ LOOP

XRA A

LOOP1: ADD B

DCR C

JZ LOOP

JMP LOOP1

LOOP: STA 8502

RST 1

INPUT:

OUTPUT:

RESULT: Thus the program was executed successfully using 8085 processor simulator.

8-BIT DIVISION

EXP NO: 4

AIM:

To write an assembly language program to implement 8-bit division using 8085 processor.

ALGORITHM:

- Start the program by loading a register pair with the address of memory location.

- Move the data to a register.

- Get the second data and load it into the accumulator.

- Subtract the two register contents.

- Increment the value of the carry.

- Check whether the repeated subtraction is over.

- Store the value of quotient and the reminder in the memory location.

- Halt.

PROGRAM:

LDA 8501

MOV B, A

LDA 8500

MVI C,00

LOOP:CMP B

JC LOOP1

SUB B

INR C

JMP LOOP

STA 8503

DCR C

MOV A, C

LOOP1: STA 8502

RST 1

INPUT:

OUTPUT:

RESULT: Thus the program was executed successfully using 8085 processor simulator.

16-BIT ADDITION

EXP NO: 5

AIM:

To write an assembly language program to implement 16-bit addition using 8085 processor.

ALGORITHM:

1)	Start the program by loading a register pair with address of 1st number.

2)     Copy the data to another register pair.

3)      Load the second number to the first register pair.

4)      Add the two register pair contents.

5)      Store the result in memory locations.

6)      Terminate the program.

PROGRAM:

LHLD 2500

XCHG

LHLD 2502

DAD D

SHLD 2504

HLT

INPUT:

OUTPUT:

RESULT: Thus the program was executed successfully using 8085 processor simulator.

16-BIT SUBTRACTION

EXP NO: 6

AIM:

To write an assembly language program to implement 16-bit subtraction using 8085 processor.

ALGORITHM:

1)	Start the program by loading a register pair with address of 1st number.

2)     Copy the data to another register pair.

3)      Load the second number to first registre pair.

4)     Subtract the two register pair contents.

5)      Check for borrow.

6)      Store the value of difference and borrow in memory locations.

7)      End.

PROGRAM:

LHLD 2050

XCHG

LHLD 2052

MVI C,00

MOV A, E

SUB L

STA 2054

MOV A, D

SUB H

STA 2055

HLT

INPUT:

OUTPUT:

RESULT: Thus the program was executed successfully using 8085 processor simulator.

16-BIT MULTIPLICATION

EXP NO: 7

AIM:

To write an assembly language program to implement 16-bit multiplication using 8085 processor.

ALGORITHM:

- Load the first data in HL pair.

- Move content of HL pair to stack pointer.

- Load the second data in HL pair and move it to DE.

- Make H register as 00H and L register as 00H.

- ADD HL pair and stack pointer.

- Check for carry if carry increment it by 1 else move to next step.

- Then move E to A and perform OR operation with accumulator and register D.

- The value of operation is zero, then store the value else go to step 3.

PROGRAM:

LHLD 2050

SPHL

LHLD 2052

XCHG

LXI H,0000H

LXI B,0000H

AGAIN: DAD SP

JNC START

INX B

START: DCX D

MOV A,E

ORA D

JNZ AGAIN

SHLD 2054

MOV L,C

MOV H,B

SHLD 2056

HLT

INPUT:

OUTPUT:

RESULT: Thus the program was executed successfully using 8085 processor simulator.

16-BIT DIVISION

EXP NO: 8

AIM:

To write an assembly language program to implement 16-bit divided by 8-bit using 8085 processor.

ALGORITHM:

- Read dividend (16 bit)

- Read divisor

- count <- 8

- Left shift dividend

- Subtract divisor from upper 8-bits of dividend

- If CS = 1 go to 9

- Restore dividend

- Increment lower 8-bits of dividend

- count <- count - 1

- If count = 0 go to 5

- Store upper 8-bit dividend as remainder and lower 8-bit as quotient

- Stop

PROGRAM:

LDA 8501

MOV B,A

LDA 8500

MVI C,00

LOOP:CMP B

JC LOOP1

SUB B

INR C

JMP LOOP

STA 8503

DCR C

MOV A,C

LOOP1: STA 8502

RST 1

INPUT:

OUTPUT:

RESULT: Thus the program was executed successfully using 8085 processor simulator.